English

Instruction and Operation Manual

SensSee

Smart-phone App for Air and Gas Flow Meter

Dear Customer,

Thank you for choosing our product.

Please read the manual in full before you start up the device and carefully observe the instructions stated. The manufacturer cannot be held liable for any damage which occurs because of non-observance or non-compliance with this manual.

Should the device be tampered with in any manner other than a procedure which is described and specified in the manual, the warranty is void and the manufacturer is exempt from liability.

The device is intended exclusively for the applications described in this manual.

Atlas Copco offers no guarantee of suitability for any other purpose. Atlas Copco is also not liable for consequential damage resulting from the delivery, capability or use of this device.

Revision: 2025-3

Last modifications: May, 2026

Table of contents

1 Registered trademarks	4

2 Application	5

2.1 System requirements	5

2.2 Operating requirements	5

3 Features	6

4 Installation and connection	6

4.1 Install the App	6

4.2 Connect to the Flow Meter	6

4.3 Enable the configuration	7

5 Menus	8

5.1 Online	8

5.2 Settings	9

5.3 System	9

6 Settings	10

6.1 Flow	10

6.2 Gas type	11

6.3 Units	11

6.3.1 Flow units	11

6.3.2 Pressure units	11

6.3.3 Temperature units	11

6.3.4 Consumption units	12

6.4 Reference conditions	12

6.5 Factory settings	12

6.6 Consumption counter settings	13

6.7 Output settings	13

6.8 Calibration	15

6.8.1 Calibration type	15

6.8.2 Calibration parameters	15

7 System	15

7.1 Sensor information	15

7.2 Language	16

7.3 App version	16

8 Maintenance	16

# Registered trademarks

| Trademark | Trademark owner |
| --- | --- |
| MODBUS® | Modbus Organization, Hopkinton, USA |
| HART® | HART Communication Foundation, Austin, USA |
| Android™, Google Play | Google LLC |

# Application

The SensSee App enables you to view measurement readings and change settings for flow meters through Android or iOS devices.

The SensSee App supports the following flow meters:

- FL S93 T, FL S185 T, FL S224 T

- FL S200 P, FL S260 P=

- FLI Dxx C (Dxx = D08, D15, D20, D25, or D32) –
FLI Dxx A (Dxx = D15, D20, D25, D32, D40, D50, D65, or D80)